Annual Report: Carbon Storage

30 September 2012

NETL Technical Report Series

NETL-TRS-Carbon Storage-2012

Disclaimer

This report was prepared as an account of work sponsored by an agency of the United States Government. Neither the United States Government nor any agency thereof, nor any of their employees, makes any warranty, express or implied, or assumes any legal liability or responsibility for the accuracy, completeness, or usefulness of any information, apparatus, product, or process disclosed, or represents that its use would not infringe privately owned rights. Reference therein to any specific commercial product, process, or service by trade name, trademark, manufacturer, or otherwise does not necessarily constitute or imply its endorsement, recommendation, or favoring by the United States Government or any agency thereof. The views and opinions of authors expressed therein do not necessarily state or reflect those of the United States Government or any agency thereof.

Cover Illustration: High resolution images of an Eau Claire shale/mudstone core sample.

Suggested Citation: Brian Strazisar, Technical Coordinator, George Guthrie, Focus Area Lead, Carbon Storage Annual Report, NETL-TRS-Carbon Storage-2012, NETL Technical Report Series, U.S. Department of Energy, National Energy Technology Laboratory, Pittsburgh, PA, 2012; p 36.

An electronic version of this report has been uploaded to the Office of Science, Technology, and Industry web site at: http://www.osti.gov/bridge/

Annual Report: Carbon Storage

Office of Research and Development, National Energy Technology Laboratory

NETL-TRS-Carbon Storage-2012

30 September 2012

NETL Contacts:

Brian Strazisar, Technical Coordinator

George Guthrie, Focus Area Lead

This page intentionally left blank

Table of Contents

1.0	EXECUTIVE SUMMARY	1

2.0	TECHNICAL HIGHLIGHTS AND KEY RESULTS	6

Project Overview	6

Task 1.0 Project Management	7

Task 2.0 Multiphase Flow in Reservoirs and Seals	7

Task 3.0 Fundamental Processes and Properties	13

Task 4.0 Estimates of Storage Potential	15

Task 5.0 Verifying Storage Performance	21

Task 6.0 Geospatial Data Resources	26

3.0	TECHNOLOGY TRANSFER	28

List of Figures

Figure 1: Sample from Vermillion County (5,806 ft.), Indiana before and after 6 months in CO2/brine, permeability reduced by 50%	7

Figure 2: (a, b) High resolution images of the as-received core, (c) grayscale cross-section of the core reconstructed from medical CT scanner images, (d) plot of the dual-energy density derived from CT scans, and (e, f) two plots of the elemental make-up of this core from the MSCL	11

Figure 3: Methods Comparison for Storage Resource Assessments	19

Acronyms, Abbreviations, and Symbols

| Acronym | Descriptive Name |
| --- | --- |
| AGU | American Geophysical Union |
| AMD | Accelerated Materials Development |
| ANN | Artificial Neural Network |
| ARRA | American Recovery and Reimbursement Act |
| AVAz | Amplitude Variation with Azimuth |
| AVO | Amplitude Variation with Offset |
| BGR | Federal Institute for Geosciences and Natural Resources |
| CAS | Chinese Academy of Sciences |
| CMU | Carnegie Mellon University |
| CRBG | Columbia River Basalt Group |
| CSLF | Carbon Sequestration Leadership Forum |
| DOE | Department of Energy |
| EDX | Energy Data eXchange |
| EOR | Enhanced Oil Recovery |
| EPA | Environmental Protection Agency |
| GIS | Geographic Information System |
| GWPC | Groundwater Protection Council |
| IC | Inorganic Carbon |
| ICMI | Industrial Carbon Management Initiative |
| ITES | Information Technology Engineering and Support |
| LBNL | Lawrence Berkeley National Laboratory |
| NATCARB | National Carbon Sequestration Database and Geographic Information System |
| NETL | National Energy Technology Laboratory |
| ORD | Office of Research and Development |
| OTS | Octadecyltrichlorosilane |
| Penn State | The Pennsylvania State University |
| PFT | Perfluorocarbon Tracers |
| PI | Principal Investigator |
| Pitt | University of Pittsburgh |
| PNGE | Department of Petroleum and Natural Gas Engineering |
| PNNL | Pacific Northwest National Laboratory |
| PPAL | Precision Petrophysical Analysis Laboratory |
| QMS | Quadrupole Mass Spectrometer |
| RCSP | Regional Carbon Sequestration Partnerships |
| R&D | Research and Development |
| REE | Rare Earth Element |
| RPSEA | Research Partnership to Secure Energy for America |
| RUA | Regional University Alliance |
| SCC | Strategic Center for Coal |
| SEM | Scanning Electron Microscopy |
| UIC | University of Illinois at Chicago |
| URS | URS Corporation |
| USGS | United States Geological Survey |
| WVU | West Virginia University |
| XANES | X-ray Absorption Near Edge Structure |
| XRD | X-ray Diffraction |

# EXECUTIVE SUMMARY

This project consists of a diverse set of research activities performed by the National Energy Technology Laboratory (NETL), Office of Research and Development (ORD), along with its partners in the Regional University Alliance (RUA) to fill multiple research needs within Carbon Storage. Activities include laboratory experimentation, field work, and numerical modeling. The work is divided into five theme areas (or first level tasks) that each address a key research need: Flow Properties of Reservoirs and Seals, Fundamental Processes and Properties, Estimates of Storage Potential, Verifying Storage Performance, and Geospatial Data Resources. The project also includes a project management effort which coordinates the activities of all the research teams.

Task 2.0 addresses the geomechanical and flow properties of rock formations (both reservoirs and seals) that affect the storage capacity, injectivity, and permanence of CO2 storage. CO2 injection can introduce significant perturbations to the chemical, biological, and geomechanical subsurface environment that can initiate a range of processes, including dissolution/precipitation of minerals and fracturing of rocks. Laboratory and numerical studies are being conducted to understand the impacts of CO2-brine-rock interactions on chemistry and microbial processes, shale porosity and permeability, and alteration of geomechanical properties on storage formations, seals, and long-term performance. The analyses and modeling are being conducted on reservoir and seal samples from Regional Carbon Sequestration Partnerships (RCSP) field sites, benefitting both the RCSP and Core Research Program goals for CO2 storage permanence through improved storage potential assessments and predictions of subsurface behavior.

- Two 6-month experiments were completed using Mt. Simon, Illinois, samples (one sandstone and one shale) at 85°C and 3,500 psi of CO2 and brine. Micro CT scan, CT scan, X-ray diffraction (XRD), scanning electron microscopy (SEM), petrography, brine, porosity, and permeability analyses were performed before and after long-term exposure to CO2 and brine under storage conditions. The shale sample showed a 30% increase in permeability. Petrographic analysis suggested an increase in porosity. Preliminary permeability measurements obtained from the Vermillion sandstone sample showed a 50% decrease. Results show CO2 injection into samples from the same formation, but dissimilar textural and mineral compositions can have significant and variable affects on overall permeability and porosity which impact reservoir performance.

- Petrography and SEM performed on the Columbia River Basalt Group (CRBG) cores revealed the presence of thick iron and magnesium carbonate lining the vugs in basalts exposed to supercritical CO2 (scCO2) and microorganisms. SEM also demonstrated the apparent formation of biofilms in fissures of the cores incubated with microorganisms.

- The evolution of the native microbial community to a gradient of CO2 concentrations over 56 days was fully characterized in samples collected from the potential CO2 storage site in the Arbuckle saline aquifer in Wellington, Kansas. Results showed a decrease in diversity and cell number with increasing CO2 exposure. Halotolerant microorganisms capable of biomineralization appeared most adaptable to CO2 exposure.

- The Precision Petrophysical Analysis Laboratory (PPAL) in the Department of Petroleum and Natural Gas Engineering (PNGE) at West Virginia University (WVU) was completed and tested for porosity and permeability testing up to 10,000 psi. The permeability and porosity measurements on one outcrop sample have been completed. The results suggest that the permeability and porosity are sensitive to net stress. The reduction in permeability with net stress was found to be irreversible (Hysteresis effects) while the reduction in porosity was found to be reversible.

- Experiments to characterize CO2 interactions with silicate minerals common in reservoir and caprock found that a mineral carbonate phase formed in the smectite swelling clay under the high-pressure CO2 (supercritical and liquid CO2). The carbonate phase is strongly dependent on the fluid phase and the exposure duration.

- Simulations of CO2 leakage into the vadose zone in the San Juan basin were continued to determine retention, leakage rates, and influencing parameters.

- An experimental facility was prepared for threshold pressure tests on the Upper Devonian shales, like the Huron, that will act as seals on any CO2 gas injected into the Marcellus Shale.

- Detailed analysis was performed on core from the FutureGen 2.0 site, including full analysis with the multisensor core logger and medical and industrial CT scans of the core. The plan has been developed to perform a series of experiments within the industrial scanner with a fractured sandstone core to analyze the permeability of the fracture under various confining pressures and relate these flow properties to the changes in geometry.

- Reservoir scale simulations were completed to assess the effect of spatially-variable relative permeability curves and residual saturations on the shape and extent of a CO2 plume. Initial results show that the CO2 storage process is significantly controlled by the formation relative permeability profile in deep saline aquifers, especially during the injection period. Reservoir scale simulations were performed to assess the effect of spatially-variable relative permeability curves, residual saturations on the shape and extent of a CO2 plume, and amount of CO2 residually trapped.

Task 3.0 addresses the fundamental processes and properties of geologic formation materials that significantly impact the ability to store CO2 for long periods of time. Work continued on mineral reaction kinetics to understand the geochemical effects of CO2 injection, to provide an estimate of potential variance in kinetic (or thermodynamic) data derived from gas-fluid-mineral interaction experiments using different experimental approaches in a variety of laboratories, to validate kinetic data for three mineral dissolution reactions at relevant in situ pressure and temperature conditions, to strengthen collaboration among experimental laboratories around the world, and to streamline experimental programs for gas-fluid-mineral reaction studies.

- NETL participated in an international inter-laboratory study on the interactions of CO2-fluid-mineral systems. The project engaged more than 14 national and academic laboratories worldwide and addressed geochemical issues related to geological storage of CO2. The project was organized and led by the Federal Institute for Geosciences and Natural Resources (BGR), Hannover, Germany, and the United States Geological Survey (USGS), Menlo Park, California.

- A prototypical caprock composition was developed for use as input for modeling to be performed in FY12. The new mineral composition of shale, close to that of Marcellus Shale, was defined for model calculations. The new composition is characterized by high content (15 mass %) of Fe-Mg chlorite. The free energy of chlorite at model TP (348.15 K, 30 MPa) was calculated. The new series of reactive diffusion models simulating the reaction and transport of scCO2 into a baseline-composition sandstone, shale interface, was started.

- Research on fabricating and testing an Ag2O/Ag electrochemical sensor for high-temperature pH measurements in brines was initiated. Thermodynamic calculation of the cell and diffusion potential was performed at 25°C and compared to experimental results. A difference between the experimental and theoretical values was found to be less than 3 mV. An IrOx-based metal oxide pH sensor was fabricated. Initial tests in NaCl(aq) solutions with concentrations up to 3 mol/kg showed a significant promise when compared to a commercial glass pH sensor and a homemade silver/silver oxide pH sensor.

- Measurements of CO2 solubility were completed in both NaCl(aq) and CaCl2(aq) solutions with ionic strength from 0 to 6 mol/kg at temperatures of 50°, 100°, and 150°C and pressure of 150 bar. The solubilities of CO2 in NaCl(aq) were systematically lower, up to about 3 percent, than that observed in CaCl2(aq) for the same temperature, pressure, and concentration.

- The reactive diffusion modeling of scCO2 phase storage at baseline sandstone (2.5 m)/shale (5 m) interface was carried out from the beginning of the process to 669 years. At this time of observance, the porosity at the sandstone/shale contact completely closed.

In Task 4.0, NETL continued to study the fundamental process and properties associated with carbon storage. This work is accomplished by continuing to investigate the methodology for assessment of unconventional systems for the estimates of storage capacity, evaluating the methodology for Enhanced Oil Recovery (EOR) CO2 storage resource assessments, a comparison of methodologies for regional and site specific assessments, and an assessment of the impact of pressure buildup and mitigation on storage capacity.

- Reservoir simulation using a history-matched model and existing production data were developed to estimate the storage resource in a select region of the San Juan Basin coal seam. Model runs using different coal seam swelling parameters showed that although coal swelling and shrinking are important on a wellbore scale for CO2 injectivity into coal seams, these effects are minor on a basin-scale for CO2 storage resource estimates in coal. As such, recommendations from the model output are that coal swelling and shrinkage factors are not necessary for CO2 storage resource estimates. In fact, incorporating swelling and shrinking needlessly complicates the storage estimate procedure, and adding these parameters is not appropriate for basin-scale estimates.

- A draft technical report was prepared comparing CO2 storage resource estimates for multiple sites using NETL’s and several other methodologies. Most of the open system methodologies gave median results that were well within the uncertainty bounds of the others, suggesting that the uncertainty accounted for within each methodology is higher than the variability between the methodologies.

- A comprehensive study was performed on the use of brine extraction to mitigate pressure 
build-up as a limiting factor for storage capacity. The study determined effects of injector type, pattern type, and pattern size on the cumulative injection amount and the injection efficiency. The simulation showed that the regular four-spot injection pattern with horizontal injector is the most effective selection in this CO2 storage project. The simulation work also determined the optimal pattern size and the injection rate of the four-spot injection pattern.

- An artificial neural network was developed to understand the nature of the existing relationships among the reservoir properties, engineering design parameters, and the desired project performance in the CO2 geology storage process as implemented in the Mt. Simon sandstone formation. The preliminary network topologies produced reasonable results as compared to 
high-fidelity numerical simulation results much more rapidly.

Task 5.0 addresses both technology development and modeling to optimize monitoring networks for tracking the fate of subsurface CO2. The task includes monitoring the near surface, groundwater, and deep subsurface for direct and indirect indications of fluid migration. Methods to quantitatively measure isotopic signatures, rare earth element profiles, and microbiological indicators are being developed and evaluated for their ability to signal CO2 intrusion. A model linking the vadose zone and atmosphere to simulate migration of CO2 is being developed to investigate coupled effects and predict locations of high probability CO2 leakage. A technique to track CO2 and pressure plumes in the deep subsurface using existing geophysical technologies, modeling, and laboratory experiments is being developed to enhance interpretation of field data for passive and active seismic monitoring and surface deformation measurements.

- Work was completed on a trace metal distribution and speciation study combining field measurements from a natural analog site with laboratory experiments and a reactive transport model. The results of this work are significant. The geologic transport of trace metals to the shallow subsurface by brine or CO2 intrusion is critical to assessing environmental impacts, and the effective retardation of trace metal transport is an important parameter to estimate and is dependent on multiple coupled reactions.

- Laboratory experiments on retention of tracers under both moist and dry conditions were completed. The main conclusion is that under dry conditions the tracers are strongly retained by geologic media, but that retention is reversed upon reintroduction of moist or humid conditions.

- Laboratory measurements were made on seismic wave properties on a core from a carbonate reservoir from an EOR site. Properties were measured under oil, CO2, and water saturated conditions.

- A study of high-quality 3D seismic survey in a fractured reservoir in the Fort Worth basin onshore Texas revealed that limited angle stack amplitude in the faulted areas is different from that in the unfaulted areas. These findings suggest that Amplitude Variation with Offset (AVO), Amplitude Variation with Azimuth (AVAz), and Spectral Decomposition data can find new applications to detect sub-seismic scale fractures. These results, which remain to be further tested and substantiated by more well bores and experiments, will have important implications for the evaluation of CO2 storage capacity and migration pathways and retention permanence in fractured and brine-saturated reservoirs.

- Generated initial uranium isotope data (238U/235U and 234U/238U) on high-CO2 natural analog samples from Chimayo, New Mexico, where mantle CO2 has impacted shallow groundwater. The data shows a significant spread in 238U/235U ratios, possibly related to redox conditions in the groundwater.

- Simulations for the pressure-history effect were performed on the seepage anomalies at the Mammoth Mountain. The pressure-history (temporal variation) effect was found to be much more effective than the wind-topography interaction (spatial variation) effect in the current settings, but both effects are detectable.

- A new C++ algorithm was developed to the computer 3D seismic curvature ad flexure attribute using a new innovative complex trace method. This algorithm can be used in combination with the waveform de-convolution and waveform model regression algorithm to better detect fractures.

- Ant-tracking post-stack seismic processing was applied to identify fault and fracture zones using the In Salah dataset.

Under Task 6.0, NETL-ORD will maintain and grow the National Carbon Sequestration Database and Geographic Information System (NATCARB) as a national geospatial data management and assessment portal. This work includes development of advanced tools and layers for geospatial display, interpretation, and analysis of storage datasets, including a new NATCARB viewer using a user-friendly MapViewer interface and development of Geodatabase Templates to facilitate the collection and exchange of data for sources and storage formations. In addition, work under this task shall include supporting the development of the next edition of the Carbon Storage Atlas IV.

- Improved tools include a new NATCARB viewer with a user-friendly MapViewer interface, and development of Geodatabase Templates. The data for a new version of the North America Carbon Atlas (Atlas IV) is complete. The new NATCARB interface was rolled out at the Strategic Center for Coal (SCC) Carbon Storage Review meeting in Pittsburgh in August. Data supplied by RCSPs and individual site characterization and field demonstration project data funded under the American Recovery and Reimbursement Act (ARRA) are being prepared for transfer into the NATCARB database. The NATCARB database will be housed within the Energy Data eXchange (EDX) data warehouse for improved searchability and more secure storage of foundation datasets. Getting an additional mirror server in place at WVU for NATCARB is in discussion.

- The full EDX Version 1 was released online in July 2012 at http://edx.netl.doe.gov. The appropriate security certificate for this website was updated by NETL’s cyber security group. A revised version of the factsheet was released. In response to guidance from SCC, and building on the ongoing collaboration with the Groundwater Protection Council (GWPC), GWPC and the EDX team have conferred on potential solutions for the states in relation to Class VI injection well data tracking and response. This effort has included coordination with the Environmental Protection Agency (EPA) as well.

# TECHNICAL HIGHLIGHTS AND KEY RESULTS

## Project Overview

This project consists of a diverse set of research activities performed by NETL-ORD along with its NETL-RUA partners to fill multiple research needs within Carbon Storage. Activities include laboratory experimentation, field work, and numerical modeling. The work is divided into five theme areas (or first level tasks) that each address a key research need. The project also includes a project management effort which coordinates the activities of all the research teams. The scope of each of the five research areas is described below:

- Flow Properties of Reservoirs and Seals – The storage capacity, injectivity, and permanence are all factors that are affected by the mechanical and flow properties of rock formations (both reservoirs and seals). Injection of CO2 can introduce a significant perturbation to the chemical, biological, and mechanical subsurface environment. This change can lead to a range of processes, including dissolution/precipitation of minerals and fracturing of rocks. The laboratory and numerical studies in this task are aimed at understanding these processes and their impact on storage performance. Samples for many of the subtasks are from RCSP field sites, which is mutually beneficial to the RCSP and core research programs. Benefits include more accurate models to predict subsurface behavior and to evaluate potential storage sinks. This addresses both high-level goals of 99% storage permanence and improved resource assessment accuracy.

- Fundamental Processes and Properties – There are key fundamental parameters and processes that have a potential impact on all carbon storage environments. This task is aimed at improving the quantitative understanding of the ability to model key chemical reaction kinetics and thermodynamic properties of fluids (CO2/brine). The benefits from this task are analogous to those of the Flow Properties of Reservoirs and Seals task above, but at a more universal fundamental level rather than a site-specific level.

- Estimates of Storage Potential – This task directly addresses the high-level goal of predicting storage capacity to ±30% accuracy by further developing estimation methodologies. Subtasks include comparison of existing methodologies and development of methodologies for EOR and unconventional sinks (coal seams and fractured shales).

- Verifying Storage Performance – The Carbon Storage Program has specific timeline and goals given in the program plan  about the development of technologies to track the fate of subsurface CO2 and ensure 99% storage permanence. This task addresses both the development of techniques, as well as modeling to optimize monitoring networks. This task includes monitoring the near surface, groundwater, and deep subsurface for direct and indirect indications of fluid migration.

- Geospatial Data Resources – There is a large amount of geospatial data and information collected throughout the RCSP sites and the core research and development (R&D) program. This task includes maintaining and enhancing the NATCARB database to share data with a wide range of potential users. An additional database will also be developed as a research tool internal to NETL. The benefit will allow both the public and researchers to gain maximum benefit from research results from ongoing projects throughout the country.

## Task 1.0 Project Management

## Task 2.0 Multiphase Flow in Reservoirs and Seals

### Subtask 2.1 Impact of CO2-Brine-Rock Chemistry on Storage Formations and Seals

The permeability and porosity of CO2 storage reservoirs affects the amount of CO2 that can be stored in the system, and affects pressurization fronts within the storage reservoir (because of fluid flow). This task uses experiments and numerical modeling to determine the impact of 
CO2-brine-rock interactions on flow properties.

Key Technology Accomplishments and Successes

Two long-term experiments with the Mt. Simon samples obtained from the Indiana Geologic Survey were completed. The conditions of the experiments were 85°C, 3,500 psi of CO2, and brine for 6 months. One sandstone sample was from Vermillion County, Indiana, with a depth of 5,806 ft. and the other was a shale sample from Knox County, Indiana, with a depth of 8,642.5 ft. Micro CT scan, CT scan, XRD, SEM, petrography, brine, porosity, and permeability analyses were performed before long-term exposure to CO2 and brine under storage conditions. Those measurements were repeated after the long-term experiments. Preliminary permeability measurements obtained from the Vermillion sandstone sample showed a 50% decrease.

The micro CT scanning was used to probe the sample obtained from Vermillion County, Indiana, before and after 6 months of exposure to the CO2/brine environment (Figure 1). After closely examining the selected red circle areas, it could be seen that mineral dissolution and mineral precipitation occurred after the sample was exposed to CO2/brine for 6 months. The precipitation might occur in the pore area that could block flow paths. This result may be responsible for the observed decrease of permeability. This result is significant because it may have a direct impact on long-term injectivity of the reservoir. Current results show CO2 injection into samples from the same formation but dissimilar textural and mineral compositions can have variable affects on overall permeability and porosity. An abstract in collaboration with IGS and the University of Utah was submitted to the coming American Geophysical Union (AGU) meeting.

Figure 1: Sample from Vermillion County (5,806 ft.), Indiana, before and after 6 months in CO2/brine, permeability reduced by 50%

The stability of the thin films of saline water between rocks and liquid CO2 plays an important role in CO2 injection and stability of geologically stored CO2. It is anticipated that the wetting films are stable on hydrophilic surfaces and unstable on hydrophobic surfaces. Virginia Tech will determine the stability of the wetting films formed on both hydrophilic and hydrophobic silica. To avoid building high-pressure equipment required to conduct the measurement with liquid CO2, measurements will be conducted using substitutes that are hydrophobic, such as silicone oils, carbon tetrachloride, dodecane, squalene, polybutenes, etc. In addition, a mathematical model will be developed based on the Reynolds lubrication theory and the Frumkin-Derjaguin isotherm, so that it can be used to determine the pressure requirement for CO2 injection and the stability of CO2 in saline aquifers.

Cleaned silica surfaces were hydrophobized by immersing them in freshly prepared octadecyltrichlorosilane (OTS)-in-cyclohexane solutions. The hydrophobicity of the silica surfaces, as measured by water contact angles, was controlled by varying the immersion time and OTS concentration. After the immersion, the excess OTS adhering on silanated silica surfaces was removed by rinsing the surfaces first by chloroform and subsequently by acetone. The silica plates were then blow-dried in a stream of pressurized nitrogen gas.

Water contact angles were measured using a Ramé-Hart Model 250 goniometer. Droplets of ultrapure water were placed on the silanated silica surfaces to measure equilibrium contact angles (θ). With a given surface, five different measurements were conducted at five different places and averaged.

Measurements on the wetting film were conducted using another hydrophobic liquid rather than soybean oil, namely silicone oil (polyphenyl-methylsiloxane), which is more hydrophobic than the soybean oil. Further, the silicon oil has a higher refractive index (1.50) than soybean oil (1.47), allowing sharper images to be obtained of the interference patterns (or Newton rings).

### Subtask 2.2 Impact of Microbial Processes on Storage Formations and Seals

Microbial populations that exist in the subsurface may serve a significant role in altering the flow properties through porous media. This task characterized the microbes present in CO2 storage reservoirs and examined the impact that CO2 injection could have on altering the microbial community and ultimately how rock permeability and porosity could be impacted. This task also assessed the potential for using microbes as a leakage mitigation technology.

Key Technology Accomplishments and Successes

Samples were collected from the potential CO2 storage site, the Arbuckle saline aquifer in Wellington, Kansas, and the evolution of the native microbial community to a gradient of CO2 concentrations was fully characterized for up to 56 days. Samples from an overlying EOR site, the Mississippian reservoir, were also exposed to a CO2 gradient for up to 56 days, and community response was characterized. These results were difficult to analyze because of the small concentration of DNA, and further analysis was delayed. Additional samples from a Texas EOR site, the Mirando Oil Reservoir, were also exposed to a CO2 gradient for up to 56 days and community response was characterized. Results showed a decrease in diversity and cell number with increasing CO2 exposure. Halotolerant microorganisms capable of biomineralization appeared most adaptable to CO2 exposure.

Pyrosequencing of microbial DNA from basalt cores was completed. DNA was obtained from autoclaves within CRBG. CRBG cores incubated with the Wallula formation water and scCO2 showed:

- An initial decrease in cell numbers upon exposure to the CO2; however, an increase in cell numbers after extended incubation.

- An abundance of bacteria and an absence of archaea after the CO2 exposure.

- A total of 47,249 bacterial “reads” signifying representative microbes in the pressure vessels containing basalt, scCO2, and formation water.

Petrography and SEM performed on the CRBG cores revealed the presence of thick iron and magnesium carbonate lining the vugs in basalts exposed to scCO2 and microorganisms. SEM also demonstrated the apparent formation of biofilms in fissures of the cores incubated with microorganisms.

### Subtask 2.3 Impact of CO2 on Shale Formations as Seals

Fractured shales are a potential reservoir for CO2 storage that has only recently been proposed. This task leveraged laboratory facilities and expertise of NETL’s Shale Gas Research Program that assess properties of shales affecting their feasibility as a CO2 storage option. The goal of this task is to obtain core-scale characterization of key processes leading to changes in seal properties.

Key Technology Accomplishments and Successes

Storage capacity, injectivity, and permanence are all factors related to the mechanical and flow properties of rock formations (both reservoirs and seals). Injection of CO2 can introduce a significant perturbation to the chemical, biological, and mechanical subsurface environment, leading to a range of processes, including dissolution or precipitation of minerals and fracturing of rocks. The laboratory and numerical studies in this task were aimed at understanding these processes and their impact on storage performance. The goal of this task was to obtain core-scale characterization of key processes affecting changes in flow properties.

The specific requirements of this task were to measure porosity, permeability, relative permeability to gas and liquids, and capillary entry pressure on a variety of shale samples under representative net confining stress using steady-state petrophysical core analysis equipment, Precision Petrophysical Analysis Lab (PPAL), and the visualization of pore structures and flowpaths affected by net stress, CO2, and liquids. Potential physical and chemical effects of the gas on petrophysical properties were assessed for a variety of shale lithologic types, and for caprocks sealed with either oil or water saturation in the pores. Reservoir models were used to analyze and interpret data.

A threshold pressure test is a simple measurement of capillary entry pressure on samples of shale being evaluated as seal material. The rock is saturated with liquid (water or oil) and mounted vertically into coreholders. The bottom end of the confined samples are then introduced to a limited volume of high-pressure gas, which moves into the core, displacing liquid out the top, expanding and dropping pressure. When the gas pressure reaches equilibrium with the pressure of fluid in the rock pores, the expansion stops and the pressure stabilizes. This stable pressure is then read off a pressure gauge, and is equivalent to the capillary entry pressure of the rock. A number of measurements of this type on a variety of core samples provided design parameters for how well a seal on a reservoir will retain pressure.

Accomplishments in FY12 include the completion and testing of the PPAL in the PNGE at WVU. As constructed over the past year, the device contains two coreholders capable of confining rock samples at pressures up to 10,000 psi, mounted inside a temperature controlled enclosure to create stable (non-fluctuating) gas pressures. Flow through the samples is measured by a differential pressure build-up in a small downstream line volume compared to a stable pressure in a large reference tank. The rate of pressure build-up in the small downstream volume is used to calculate flow. Porosity to gas can also be measured under confining pressure using a Boyle’s Law method with a volume-calibrated displacement pump.

The data gathering process was started to find information such as production data for building a reservoir model. A base case reservoir model was created based on an actual field, in which reservoir scale simulations were performed to assess the effect of spatially-variable relative permeability curves and residual saturations on the shape and extent of a CO2 plume.

The permeability and porosity measurements on one outcrop sample have been completed. The results suggest that the permeability and porosity are sensitive to net stress. The reduction in permeability with the net stress was irreversible (Hysteresis effects) while the reduction in porosity was reversible. In addition, the Klinkenberg effects have been investigated by measuring the permeability under different gas pressure (constant net stress). The results were used to estimate the absolute (liquid) permeability of the sample. The permeability and porosity measurements on a second outcrop core sample have been initiated.

The permeability and porosity measurements on the second outcrop core sample are in progress. The results show that the permeability and porosity are sensitive to net stress. The reduction in permeability with the net stress was found to be irreversible. The Klinkenberg effects are being investigated by measuring the permeability under different gas pressure (constant net stress). However, the low permeability of the sample has caused the experiment to proceed slowly.

### Subtask 2.4 Impact of Geomechanical Processes on Seal Integrity

Just as a reservoir with potential for carbon storage can vary significantly across and within geologic formations, so do natural seals vary significantly with location and type. As CO2 is injected into a reservoir, this changes the effective stresses felt by the cap rock for that formation. Additionally, as CO2 reaches the sealing formation, it has the potential to alter the caprock mechanical properties. As chemical and mechanical processes occur in sealing formations with fractures, these fractures may open or close, depending on the conditions. In this task, a combination of numerical modeling and measurements on field samples were used to determine the impact of geomechanical stresses on seal properties.

Key Technology Accomplishments and Successes

The network fracture model has been run for larger systems (1.44 million blocks). Looking for evidence for the validity of the model, an expert on caves said that the modeled fracture networks resemble maze-caves in limestone that are formed when acidified water flows through fracture networks. Network plots of these cave types are being analyzed to do a quantitative comparison with the model results. The one quantitative comparison completed demonstrates very good agreement. Several experts on speleogenesis have been contacted and have expressed interest in the fracture network modeling as an analog to cave morphology. Although the model describes underground fracture networks; the cave comparison is primarily being explored as a test of the model.

Core was obtained from the FutureGen 2.0 site, including the Mt. Simon sandstone (reservoir) and the Eau Claire shale/mudstone (primary seal). Detailed analysis was performed on the core from the FutureGen 2.0 site, including full analysis with the multisensor core logger and medical and industrial CT scans of the core (Figure 2). A plan was developed to perform a series of experiments within the industrial scanner with a fractured sandstone core to analyze the permeability of the fracture under various confining pressures and relate these flow property to the changes in geometry.

A 6-inch sample of the Eau Claire caprock from a depth of 3,804 ft. was obtained from the Pacific Northwest National Laboratory (PNNL) to perform the flow through tests required by this project. The following images are of (top to bottom, left to right) high resolution images of the 
as-received core, a grayscale cross-section of the core reconstructed from medical CT scanner images, a plot of the dual-energy density derived from CT scans, and two plots of the elemental make-up of this core from the MSCL.

(a)  				   (b)  				(c)

(d)  				(e)  				(f)

Figure 2: (a, b) High resolution images of the as-received core, (c) grayscale cross-section of the core reconstructed from medical CT scanner images, (d) plot of the dual-energy density derived from CT scans, and (e, f) two plots of the elemental make-up of this core from the MSCL.

### Subtask 2.5 Understanding of Multiphase Flow for Improved Injectivity and Trapping

The prediction of reservoir performance depends greatly on understanding the multiphase flow, residual saturation, and multiphase flow. This task uses numerical modeling, laboratory measurements, and field samples to focus on the key processes that will allow for more accurate prediction of capacity, injectivity, and storage permanence.

Key Technology Accomplishments and Successes

Initial studies using the Industrial CT scanner have been performed to obtain pore geometries. Cores from the Chinese Academy of Sciences (CAS) were acquired and several characterization measurements were made. Several thin sections and CT images were taken and permeability measurements were made. A relative permeability apparatus was set up and initial permeability measurements were made on these cores. A research plan was made and a schedule for continued collaboration was developed.

Reservoir scale simulations were performed to assess the effect of spatially-variable relative permeability curves and residual saturations on the shape and extent of a CO2 plume. Initially, three relative permeability profiles (three sandstones) from the six different profiles gathered during the literature review process were used in the simulations. The simulations were run for a period of 500 years for each one of the relative permeability profiles considering hysteresis. Initial results were analyzed indicating that the CO2 storage process is significantly controlled by the formation relative permeability profile in deep saline aquifers, especially during the injection period considering that hysteresis plays a very important role in residual gas trapping, leading to an immobile and spread out distribution of CO2, rather than to a structural trap of mobile CO2. The model being used for this reservoir scale simulation has been laterally homogeneous and vertically heterogeneous (Mattoon Formation, Illinois). To better grasp the understanding of the different trapping mechanisms in play (particularly residual trapping), a more complex and heterogeneous model was used (Citronelle Field, Alabama) to run reservoir scale simulations as before, but in addition to using the real sample drainage relative permeability data available, actual data from the imbibitions process was also obtained. This provides a more realistic approach about the residual saturations that considerably influence the residual trapping mechanism.

Using a UIC Coulometric Carbon Analyzer (model CM5015: CM5130 acidification module, CM5300 furnace with a CM5390 automated boat inlet), the SWy-2 Wyoming clay samples exposed to sc and liquid CO2 were analyzed. This experiment was conducted for a virgin observation. However, the results showed the exposed clay samples have inorganic carbon (IC) captured by the CO2 treatment, showing a consistent result with that of X-ray photoelectron spectroscopic experiments that were previously conducted. The experiment that was initiated for a carbonation reaction diagram is continuously being conducted. The clay samples are exposed to various CO2 conditions and analyzed using optical spectroscopes.

CO2 interactions with the silicate minerals that are common components of the hostrock and caprock within the reservoir are being investigated by Fourier transform infrared spectroscopy, X-ray photoelectron spectroscopy, and powder X-ray diffraction. To investigate the dependency of carbonation reaction on the pressure and temperature of CO2 in detail, the SWy-2 Wyoming clay was exposed to the fluid at various pressures and temperatures, as a function of exposure time. The background-subtracted carbonate band corresponding to the CO32- asymmetric stretching vibration (ν3) mode is being monitored. The overall intensity of the carbonate band gradually increased with increasing CO2 exposure time, as mentioned before. Interestingly, as the temperature of CO2 fluid increased from 45C to 90C in the range of supercritical phase, the intensity ratio of the field-splitting band centered at ~1476 cm-1 to primary band at ~1420 cm-1 of the CO32- asymmetric stretching mode gradually increased. On the other side, it is likely that the overall intensity of the CO32- asymmetric stretching band suddenly decreases at the temperature of 90C. Currently, the behavior of the temperature-pressure dependencies of the chemical reaction is being carefully investigated.

The carbonation reaction that occurs in the SWy-2 Wyoming clay by exposure of CO2 fluids at different temperature-pressure conditions was investigated using Fourier-transform infrared spectroscopy. CO2 fluids representing gas, liquid, and supercritical phases were realized to seriously affect the carbonation reaction. The appearance of the field-splitting band centered at ~1476 cm-1 and the primary band at ~1420 cm-1 of the CO32- asymmetric stretching mode showed a complex behavior in the reaction, especially at 1500 psig and 90C. And the intensity of the carbonate band at ~1420 cm-1 was dramatically changed at the pressure of 500 psig, as a function of temperature.

The behavior of the temperature-pressure dependencies of the carbonation reaction in the 
as-received SWy-2 Wyoming clay is being investigated for a potential journal publication together with the observation of the chemical reaction in the cation-exchanged SWy-2 clays 
(Ca-SWy-2 and Na-SWy-2), as well as with the observation on the STx-1b clay (Texas: 
as-received, cation-exchanged) and the SAz-2 clay (Arizona).

## Task 3.0 Fundamental Processes and Properties

### Subtask 3.1 Improved Mineral Reaction Kinetics

This task targets Geochemical Effects of CO2 Injection. The main objective of this task is to assess uncertainty on a worldwide scale associated with geochemical experiments in 
gas-fluid-mineral systems. The motivation for this study is threefold: (1) to provide an estimate of potential variance in kinetic (or thermodynamic) data derived from gas-fluid-mineral interaction experiments using different experimental approaches in a variety of laboratories, (2) to validate kinetic data for three mineral dissolution reactions at relevant in situ pressure and temperature conditions (siderite, labradorite, and illite at p=200 bars, T=80°C, CO2-saturated 2 M NaCl-brine), and (3) to strengthen the collaboration among experimental laboratories around the world and to streamline experimental programs for gas-fluid-mineral reaction studies.

Key Technology Accomplishments and Successes

NETL participated in an international inter-laboratory study on the interactions of CO2-fluid-mineral systems. The purpose of this study is to compare the variability in results of geochemical changes to water chemistry and sedimentary mineral composition between different experimental techniques and apparatuses employed by different research groups. The project engages more than 14 national and academic laboratories worldwide and is focused on addressing geochemical issues related to geological storage of CO2. The project is organized and led by BGR, Hannover, Germany, and USGS, Menlo Park, California.

NETL researchers conducted a set of three CO2-brine-mineral reactions at 80°C, 200 bar in rocking autoclave reactors. The experiments investigated the reactivity of three different minerals: illite, labradorite, and siderite with CO2-saturated synthetic brine under conditions relevant to current CO2 storage activities. Each experiment ran for several weeks with intermittent sampling of brine and post-reaction sampling of solids. Aqueous samples were analyzed to determine dissolved CO2 content, aqueous cation and anion composition, and pH. NETL researchers prepared a report detailing experimental and analytical methods and reporting results from fluid analyses. This report and solid reaction products were delivered to the Round Robin organizers in December 2011; those results will be integrated with those from other participating laboratories and the results will be distributed for collaborative evaluation/discussion and final reporting.

The kinetic database for reactive diffusion CO2 storage modeling at baseline sandstone/shale interface was added by a set of new mineral phases: pyrite, chlorite, dolomite, siderite, goethite, and magnetite with necessary parameters to calculate kinetics at 75°C and 300 MPa. The additional change with incorporation of activities of Fe3+(aq) and dissolved oxygen into kinetic subroutine of MK76 was necessary to account for pyrite dissolution/precipitation.

The numerical program MK76 was modified to allow for initial rock heterogeneity. The new initial rock setup can comprise N rock layers of different mineral compositions. A test of reactive diffusion modeling of scCO2 phase storage within a baseline sandstone (2.5 m)/shale (5 m) interface was carried out for up to 100 years using the new extended database. The initial pore brine compositions were determined by assuming chemical equilibrium of average North American oil field brine with the initial mineral assemblages of the sandstone and shale. The sandstone porous medium was filled by the scCO2 phase at 5 volume %. The reactive diffusion problem was computed at 75°C and 30 MPa for scCO2, 15 mineral phases, and aqueous brine composed of 40 species. The intensive mineral dissolution at interface was observed in simulation for calcite, chlorite, scCO2 residual phase, and sandstone oligoclase.

The reactive diffusion modeling of scCO2 phase storage at baseline sandstone (2.5 m)/shale (5 m) interface was carried out from the beginning of the process to 669 years. At this time, the porosity at the sandstone/shale contact was observed to be completely closed. The reactive process at the sandstone/shale interface causes complete sealing for sandstone + CO2 during 670 years. The reactive diffusion computations were carried out at 75°C and 30 MPa for scCO2, 15 mineral phases, and an aqueous brine composed of 40 species. The depth of CO2 penetration into shale after 669 years was equal to 1.1 m. In this region, the water-rock interactions were accompanied by an average decrease in shale porosity from 5% to 2.5% with complete porosity closure at the sandstone/shale contact because of the intensive precipitation of dolomite, kaolinite, albite, and ankerite.

### Subtask 3.2 Improved Prediction of Fluid Properties

Solution pH is a key parameter affecting the reactivity of CO2 saturated saline waters with formation rock, natural, and engineered seals in geologic CO2 storage conditions (potentially impacting both reaction equilibria and kinetics); uncertainty in the reliability of solution prediction in conventional reservoir models introduces significant uncertainty into the overall assessment of reservoir long-term performance and related risks. This task is aimed at making laboratory measurements to validate/improve pH predictions from geochemical models.

Key Technology Accomplishments and Successes

Measurements of CO2 solubility were completed in both NaCl(aq) and CaCl2(aq) solutions with ionic strength from 0 to 6 mol/kg at temperatures of 50, 100, and 150°C and pressure of 150 bars. The solubilities of CO2 in NaCl(aq) were systematically lower, up to about 3% than that observed in CaCl2(aq) for the same temperature, pressure, and concentration.

Measurements of scCO2 solubility are near completion in Na2SO4(aq) solutions. At 150 bars, the scCO2 solubility in Na2SO4(aq) solutions with ionic strengths ranging from 0 to 6 mol/kg were measured at temperatures of 50, 100, and 150°C. Water vapor pressure measurements were conducted to experimentally determine the activity of water in electrolyte solutions at elevated temperatures. These measurements are needed to correctly implement any Me/MeO pH sensor.

Progress towards in-situ pH measurements was made in the areas of pH sensor fabrication, reference electrode fabrication, and construction of a high-temperature and high-pressure sensor calibration cell. Fabrication of iridium-lithium oxide-based pH electrodes (IrOx) was continued, and the prepared sensors were tested in select solutions. Homemade silver-silver chloride reference electrodes were fabricated for use in high-temperature and high-pressure environments. A high-temperature and high-pressure flow cell was constructed to calibrate the IrOx at scCO2 storage conditions.

A homemade IrOx electrode was used to experimentally determine the pH of 0, 1, 2, 3, and 4 mol kg-1 NaCl(aq) in dilute HCl(aq) solutions. The pH values experimentally determined for dilute HCl(aq) solutions were within 0.1 pH units of values calculated by the Gibbs energy minimization code. An IrOx electrode potential sensitivity for oxygen-based anion species was observed in some specific cases. However, thick Nafion coatings were used to overcome this issue with some success. At 25C and 1 bar, the pH of CO2-saturated 0, 1, and 2 mol kg-1 NaCl(aq) solutions were experimentally determined. The experimentally obtained pH values were within 0.05 pH units of calculated values.

## Task 4.0 Estimates of Storage Potential

### Subtask 4.1 Methodology for Assessment of Unconventional Systems

This task involved expanding and adapting NETL-ORD’s published “Methodology for Development of Geologic Storage Estimates for Carbon Dioxide” to the regional assessment of unconventional reservoirs. The current carbon storage methodology is applicable to conventional, porous, and permeable reservoirs such as sandstones. Storage resource estimates for reservoirs, such as shales, coal seams, and basalts, needs adaptation of the assessment methodology to account for key differences in these lithologies. For example, storage assessments in shale, coal, and basalt systems will need to consider structural features such as fracture density and connectivity, as well as the potential adsorption of CO2 to mineral/coal surfaces.

Key Technology Accomplishments and Successes

The current carbon storage methodology is applicable to conventional, porous, and permeable reservoirs such as sandstones, but not to the key differences in lithology in unconventional reservoirs such as shales, coal seams, and basalts. The assessment methodology needs adaptations to account for features such as fracture density and connectivity, stress effects, and permeability changes, as well as the potential adsorption of CO2 to mineral or organic surfaces. The work was divided into two sub-subtasks, one focused on gas shales and the other on coals.

The USGS and independent assessments of natural gas resource potential in shales were used as the starting point for CO2 storage methodology. The difference in gas capacity between methane and CO2 was measured in the laboratory, and additional measurements from RCSP projects, Research Partnership to Secure Energy for America (RPSEA) projects, and other sources were also used. Based on the existing data, shales are anticipated to be able to adsorb significantly more CO2 than methane, perhaps as high as 5 to 1. Adsorption at low-pressure, and the behavior of scCO2 in the pores at high-pressure must be accounted for in any storage methodology. Complicating factors include changes in flowpaths or irreversible formation damage because of excursions to high net stress during drawdown. Strategies to mitigate such formation damage, including the potential start of CO2 injection to maintain reservoir pressure while CH4 is still being produced, will also be explored.

Accomplishments in FY12 included ongoing porosity and permeability data collection on Marcellus Shale and other samples using the PPAL in the PNGE at WVU. Because only two samples can be measured simultaneously, and temperature stability must be achieved to provide accurate reference pressures, throughput of samples in the apparatus is slower than desirable. Discussions with URS Corporation and WVU on ways to increase the analysis rate included expanding the PPAL device by adding more coreholders, building another apparatus to operate in parallel, or sending some cores out for commercial analyses using a pressure pulse technique. External core analysis will help speed up data gathering and will also provide data for comparison to the PPAL results.

Independent laboratory data on gas adsorption isotherms have been collected on a number of crushed Marcellus Shale samples under the Industrial Carbon Management Initiative (ICMI) project. These data will be used in a carbon storage methodology for gas shales. Correlations with organic content, clay, quartz, and carbonate are expected to help relate adsorption ability to geological factors, thereby improving the predictability of this parameter.

Reservoir simulation using a history-matched model and existing production data were developed to estimate the storage resource in a select region of the San Juan Basin coal seam. Impacts of coal swelling were estimated by varying the swelling parameters within the coal seam storage model. These results were then used to assess the need for incorporating swelling in CO2 coal seam storage.

Model runs using different coal seam swelling parameters showed that although coal swelling and shrinking are important on a wellbore scale for CO2 injectivity into coal seams, these effects are minor on a basin-scale for CO2 storage resource estimates in coal. As such, recommendations from the model output are that coal swelling and shrinkage factors are not necessary for CO2 storage resource estimates. In fact, incorporating swelling and shrinking needlessly complicates the storage estimate procedure, and adding these parameters is not appropriate for basin-scale estimates.

### Subtask 4.2 EOR CO2 Storage Resource Assessment

The current methodology to assess CO2 storage capacity in oil-bearing reservoirs available through EOR allows for alternative approaches: volumetric and production-history-based methodologies. In this task researchers evaluated the appropriateness of these methodologies through a case study using a type field and production datasets. A more explicit description of assessment methodologies will be prescribed, including recommendations for incorporation of uncertainties about reservoir properties and storage efficiency. The initial effort focused on evaluation of the volumetric assessment methodology for a conventional petroleum reservoir employing EOR for tertiary oil recovery; in forthcoming fiscal years researchers will continue evaluating the same type field using a production-history-based methodology and compare/contrast performance of both methodologies. Results of this evaluation will be published to benefit future resource assessments by RCSPs and other interested entities.

Key Technology Accomplishments and Successes

Methodologies spelled out in the Carbon Sequestration Atlas are intended to generate high-level, quantitative estimates of CO2 storage resource. Saline reservoir methodology and coal seam methodologies have been revised to account for the subsurface volumes that may be technically available for CO2 storage through the application of a set of efficiency terms (fraction of total pore volume amenable to geologic storage) and displacement terms (fraction of pore volume adjacent to an injector that can be technically accessed for CO2 storage).

As currently written, the resource assessment methodology for CO2-EOR (and CO2-EGR, though that is not being considered in the scope of this subtask) describes two methodologies that may be applied by RCSPs: a volumetric approach and a production-based approach. Neither of these approaches is explicitly described, nor is guidance given for how to account for uncertainty in the assessments as has been done for coal and saline formations. While RCSPs have not been required to disclose in the Atlas the approach that was employed to develop their resource assessments, anecdotal evidence suggests that most, if not all, have used a form of a production-based approach in favor of a volumetric approach. The case may be that even if all RCSPs are applying a production-based methodology for CO2-EOR resource assessment, the specific assessment choices are inconsistent between groups.

This work is, therefore, intended to generate a draft revised methodology to facilitate generation of a quantitative, high-level CO2 storage resource assessment methodology that is clearly defined, takes into account uncertainties in input parameter values that propagate to uncertainties in the related resource assessments, and is accessible to RCSP Principal Investigators (PIs) and the general public. The proposed methodology will adopt the production-based approach. This effort is not intended to develop a resource assessment, as that is the purview of the RCSPs, and a data-driven exercise for which NETL-ORD currently lacks robust data.

Toward that goal, efforts have focused on (1) initiating development of a conceptual model for characterization of CO2 storage potential of a CO2-EOR field, (2) coordinating the effort of this subtask with similar efforts ongoing in other offices of NETL, (3) evaluating modeling tools that are freely available to develop high-level assessments of CO2-EOR production/CO2, and (4) initiating consideration of how to treat the problem stochastically.

Researchers initiated the development of a draft report describing the identified issues with the methodology for oil and gas field CO2 storage that is published in the Atlas III. Specific topics that have been identified to be described more explicitly include effective porosity, reservoir volume factor, CO2 density at standard conditions, and the CO2 storage efficiency term. Approaches for determining efficiency of CO2 storage in oil formations using reservoir performance screening tools and numerical reservoir simulators are outlined. In the case of screening tools an example is given where a set of modeling input parameters are enumerated and discussed with respect to how they might best be treated as either deterministic or stochastic parameters and differentiation between data, model, and scenario uncertainty. The use of Dykstra Parsons coefficient to characterize permeability heterogeneity and uncertainty related to that parameter is discussed. Insights from CO2 storage in saline formations with respect to prediction of formation temperature, pressure, and porosity as a function of depth are recommended for application in CO2-EOR methodology. CO2 injection operational characteristics and well configuration parameters are discussed as scenario choices to be evaluated through sensitivity analysis, and an approach for considering storage resource impact from formation pressurization with CO2 beyond CO2-EOR flood business-as-usual is described.

### Subtask 4.3 Comparison of Methodologies for Regional and Site Specific Assessments

This task targets the Geologic Storage Research Goals and Proposed Timeline to demonstrate enhanced CO2 trapping and storage resource at precommercial scale and demonstrate the ability to predict CO2 storage capacity within 30 percent accuracy. The main deliverable from this task is to feature a page in the Carbon Storage Atlas IV that compares alternative methodologies (USGS, DOE, and CGSS) on a basin-scale such as the Illinois basin or sub-regional-scale for a formation such as the Oriskany formation.

Key Technology Accomplishments and Successes

Comparison of CO2 storage resource estimates was completed using the USGS, Carbon Sequestration Leadership Forum (CSLF), CGSS, and DOE methodology and submitted to SCC to review and use as a potential Feature Page in Atlas IV. A draft technical report was prepared.

In this study, high-level CO2 methodologies were compared for development of geologic storage estimates for CO2 in saline formations to assess the uncertainty associated with various methodologies. The purpose was not to validate data sets, endorse methodologies, or release CO2 estimates for specific formations. The purpose was to compare methodologies using the same input data to assess the uncertainty associated with various methodologies. Specifically, five publically-available, transparent methodologies were applied to 16 saline formations. The methodologies applied are:

- CO2 Storage Capacity Estimation: Methodology and Gaps (Bachu, Bonijoly, et al., 2007).

- A Method for Quick Assessment of CO2 Storage Capacity in Closed and Semi-closed Saline Formations (Zhou, Birkholzer, et al., 2008).

- A Probabilistic Assessment Methodology for the Evaluation of Geologic Carbon Dioxide Storage (Brennan, Burruss, et al., 2010).

- U.S. DOE Methodology for the Development of Geologic Storage Potential for Carbon Dioxide at the National and Regional Scale (DOE-NETL 2006; DOE-NETL 2008; DOE-NETL 2010).

- Lifetime of Carbon Capture and Storage as a Climate-Change Mitigation Technology (Szulczewski, MacMinn, et al., 2012).

The 16 saline formations targeted were characterized by Szulczewski, et al. (Szulczewski, MacMinn, et al., 2012). While detailed information about the 16 formations is available (Szulczewski, MacMinn, et al., 2012), location names were avoided because of the inability to endorse the validity of this data set.

The following general trends were determined for the CO2 storage estimates in Figure 3 at the 50 percent probability range for open and closed systems by the methodologies [NETL Atlas I, II: Frailey 2006, 2008 and NETL Atlas III: Goodman, et al., 2011, CSLF: Bachu, et al. (2007), USGS: Brennan, et al. (2010), Szulczewski, et al. (2012), and Zhou, et al. (2008)]:

- All six methodologies fell within two standard deviations of the mean of an arithmetic averaging estimator for all 16 locations.

- The method by Szulczewski, et al. (2012), typically, reports the lowest estimates.

- The method by USGS: Brennan, et al. (2010), typically, reports the highest estimates.

- In most cases, the closed estimates by Zhou, et al. (2008), are similar to the estimates provided by Szulczewski, et al. (2012).

- In most cases, the estimates by NETL Atlas I, II: Frailey 2006, 2008 and NETL Atlas III: Goodman, et al. (2011), CSLF: Bachu, et al. (2007), generally fall between the estimates by USGS: Brennan, et al. (2010), and Zhou, et al. (2008).

Several different resource estimation methodologies were applied to a wide range of different site types. As is typical for these types of estimates currently for carbon storage in saline fields, the data sets were very sparse. Most of the open system methodologies gave median results that were well within the uncertainty bounds of the others, suggesting that the uncertainty accounted for within each methodology is higher than the variability between the methodologies. The results give a high degree of confidence that the methodologies are reasonable and that the results of applying these methodologies to a basin or larger region will give results that can be used by 
decision-makers.

Additionally, a methodology that assumes a closed system was used for comparison. The closed system estimates were consistently lower than those of the open system methodologies, but the estimated values from the closed system were also mostly well within the uncertainty bounds of the open system estimates. These estimates suggest that the differences between closed and open system estimates may have been overblown (Economides and Ehlig-Economides 2009; Xie and Economides 2009).

Also, work focused on examining detailed data for the Oriskany formation in Pennsylvania and West Virginia. Detailed spatial parameters are being collected and analyzed for this formation as regional and site-specific methodologies are applied to the Oriskany formation.

Figure 3: Methods Comparison for Storage Resource Assessments

Using temperature and pressure gradients calculated from the data and agreed upon with researchers doing related research at Carnegie Mellon University (CMU), temperature and pressure data have been generated for the top of the Oriskany formation.

The areas in which the Oriskany is at deeper than 800m below the surface are now being focused on, with this as a proxy for the conditions at which storage is possible. Work is being performed to determine where structural traps might exist to contain fluid (Disenhof).

### Subtask 4.4 Impacts of Pressure Buildup and Mitigation on Storage Capacity

Injection of CO2 into a reservoir with closed boundaries could limit capacity for CO2 storage since there is no mechanism to dissipate pressure. This effect could be especially important in the case of multiple injection wells. This task assesses the impact of pressure buildup in wells and evaluates the potential for mitigation by brine extraction.

Key Technology Accomplishments and Successes

ORD researchers assisted in the preparation of a manuscript about the effects of density changes of brine because of the dissolution of CO2. Several cases were modeled and the effect of dissolution on storage efficiency was determined. A manuscript was prepared.

To simulate the geology characteristics of the Mt. Simon sandstone formation, the entire Mt. Simon sandstone formation is divided into five different regions based on their burial depth and formation thickness.

- Section 11 Northern Illinois Basin and Western Kankakee Arch: This study area is called “shallow and thick” region with formation depth ranges from 2,500 to 5,000 ft. and formation thickness ranging from 2,000 to 2,500 ft.

- Case 2 Western Michigan Basin: This study area is called “intermediate depth and thickness” region with formation depth ranges from 5,000 to 7,000 ft. and formation thickness ranging from 1,400 to 1,500 ft.

- Case 3 Cincinnati Arch: This study area is called “intermediate depth and thin” region with formation depth ranges from 5,000 to 7,000 ft. and formation thickness ranging from 500 to 1,400 ft.

- Case 4 Northern Michigan Basin: This study area is called “extremely thin and deep” region with formation depth ranges from 7,000 to 10,000 ft. and formation thickness ranging from 100 to 500 ft.

- Case 5 Mid-Michigan Basin: This study area is called “extremely deep and intermediate thickness” region with formation depth ranges larger than 10,000 ft. and formation thickness ranges from 1,200 to 1,300 ft.

A comprehensive study was conducted on the effects of injector type (horizontal or vertical), pattern type (regular or inverted), and pattern size on the cumulative injection amount and the injection efficiency by applying Monte Carlo Simulation protocols. An attempt was made to find the optimal design criteria towards maximizing the cumulative injection amount as well as the injection efficiency.

The simulation shows that the regular four-spot injection pattern with horizontal injector is the most effective selection in this CO2 storage project. The simulation work also determined the optimal pattern size and the injection rate of the four-spot injection pattern.

In this analysis six new terminologies are introduced in discussing the design analysis. These new terminologies include Peak P90 Cumulative Injection Amount, Cumulative Injection Limiting Injection Rate, Injection Efficiency Limiting Injection Rate, Threshold Injection Rate, Threshold Cumulative Injection Amount, and Threshold Injection Efficiency.

- Peak P90 Cumulative Injection Amount - The peak point on the injection rate versus P90 cumulative injection amount curve.

- Cumulative Injection Limiting Injection Rate - The injection rate at which the P90 cumulative injection rate starts to enter plateau region at P90 cumulative injection rate versus injection rate plot.

- Injection Efficiency Limiting Injection Rate - The injection rate at which the P90 injection efficiency starts to enter the plateau region at P90 injection efficiency versus injection rate plot.

- Threshold Injection Rate - The smaller value between cumulative injection limiting injection rate and the injection efficiency limiting injection rate. This term is an indication of the injection rate at which the CO2 injection has the most effective performance in view of the cumulative injection and the injection efficiency.

- Threshold Cumulative Injection Amount - The corresponding P90 cumulative injection amount of CO2 at the threshold injection rate.

- Threshold Injection Efficiency - The corresponding P90 injection efficiency at the threshold injection rate.

In CO2 injection into brine formations, the resulting formation pressure after the stabilization of CO2-brine system is a significant parameter to consider. In designing the injection operation, one should make sure that the reservoir pressure after stabilization is lower than the initial reservoir pressure to minimize the risk of CO2 leakage and upwards migration. This study applies Monte Carlo simulation protocol to track the stabilized injection block pressure after the injection is stopped. A new term, pressure depletion ratio, is defined to evaluate the influence of CO2 injection to formation pressure after stabilization. The design should not solely chase for high cumulative injection efficiency values. The pressure depletion ratio has to be less than one to ensure that the injection process is conducted in consideration of a safety factor. An analysis protocol involving Artificial Neural Network (ANN) application is considered to be helpful to minimize the number of time demanding compositional numerical simulations.

ANN was developed to understand the nature of the existing relationships among the reservoir properties, engineering design parameters, and the desired project performance in the CO2 geology storage process as implemented in the Mt. Simon sandstone formation. When ANN is trained by enough input parameters and corresponding output parameters, the network can be treated as a trustable expert system and can be used to generate output files for a large number of input data rapidly. By utilizing the expert system, a large number of studies can be generated for the Monte Carlo simulation analysis and hence high resolution pressure depletion ratio and injection efficiency surfaces can be built.

Two networks are developed in this study. First is the forward solution network. The inputs of the forwards solution network are reservoir properties and engineering design parameters. The outputs are cumulative CO2 injection volume, cumulative brine production, and the well block pressure values at the end of a stabilization period. The second network is the inverse solution network. The network outputs the required engineering design parameters when the expected project performance parameters and reservoir properties are input. The preliminary network topologies developed for the forward-looking and inverse-looking networks produce reasonable results as compared to high-fidelity numerical simulation results more rapidly.

## Task 5.0 Verifying Storage Performance

### Subtask 5.1 Natural Geochemical Signals to Monitor Leakage to Groundwater

Detection of CO2 migration into shallow groundwater aquifers is important both to assess storage permanence and to evaluate impacts on water resources. This task seeks to develop methodologies for quantitative measurement of isotopic signatures, rare earth element (REE) profiles, and microbiological indicators to determine their ability to signal CO2 intrusion. The ultimate objective is to provide a suite of techniques that, when used alone or in combination, can show CO2 losses in excess of the 99% target over 100 years, or 10,000 tons in one year.

Key Technology Accomplishments and Successes

Trace metal distribution and speciation studies were continued at the Argonne National Laboratory in October 2011. This trip used micro X-ray fluorescence (-XRF) mapping and micro X-ray Absorption Near Edge Structure (XANES) spectroscopy to evaluate distribution and speciation of iron and arsenic in both natural analog sediments (Chimayo) and CO2 storage reservoir and caprock materials from the Southeast Regional Carbon Sequestration Partnership (SECARB) site. Qualitative data shows a strong trend between arsenic and iron in many of the sediment samples, both in oxidized and reduced systems.

The trend of arsenic association with iron, as determined from the -XRF mapping results, was used to help construct input parameters for modeling of trace metal transport in the Chimayo natural analog site. After extensive characterization of the sediment mineralogy from the Chimayo site, arsenic is likely considered an adsorbed species associated with iron-rich clay materials in the sediments. In addition, previous batch laboratory experiments performed at NETL with Chimayo sediments and groundwater show that pH decreases as CO2 is introduced into the system and buffered by calcite. The introduction of CO2 causes an immediate increase in As solution concentration, which then decreases over time. This information was used by collaborators at the Los Alamos National Laboratory to develop a robust geochemical model to simulate these batch experiments and characterization results. Based on modeling, illite precipitation allows for additional sorption sites to form as a function of time resulting in a slow decrease in As concentration (as was shown in the batch experiments). This mechanism can be viewed as trace metal “scavenging” because of sorption caused by secondary mineral precipitation. Because deep geologic transport of trace metals to the shallow subsurface by brine or CO2 intrusion is critical to assessing environmental impacts, the effective retardation of trace metal transport is an important parameter to estimate and is dependent on multiple coupled reactions.

The results of this work are significant. The geologic transport of trace metals to the shallow subsurface by brine or CO2 intrusion is critical to assessing environmental impacts, and the effective retardation of trace metal transport is an important parameter to estimate and is dependent on multiple coupled reactions. At the field scale, As mobility is retarded because of the influence of sorption reactions, which can affect environmental performance assessment studies of a storage site. Further work is planned in this area to better understand the sorption mechanisms and also the role of iron in the sorption reactions. Quantitative analysis of spectroscopy data is ongoing and will be used to help determine the role of sorption reduction for both the Chimayo groundwater system and also for deeper saline reservoir and caprock systems.

An Sr isotope analysis was carried out on three sets of water collected along a transect of a 
high-CO2 natural analog site associated with AMD. The Sr isotopic signature of the discharges from the mine portal are consistent with the development of similar alkaline waters as a result of carbonate mineral dissolution enhanced by subsurface cation exchange reactions between mine waters and clay minerals. The mine portal water sample from December 2010 had a Sr isotopic composition lower than that collected the following July, consistent with carbon isotopic data that suggest a greater contribution from limestone dissolution in December.

Several isotope systems are being considered for planned CO2-H2O-rock experiments being carried out by collaborators at NETL and CMU. Development of clean laboratory separation chemistry techniques has been initiated for the following relevant isotope systems: strontium (Sr), lithium (Li), iron (Fe), copper (Cu), and neodymium (Nd). Initial results from the experiments provided information as to which combination of these systems will best be applied to understand fluid-rock interactions.

Initial uranium isotope analyses (238U/235U and 234U/238U) were completed on natural analogue samples from Chimayo, New Mexico, where mantle CO2 has impacted shallow groundwater. The data shows a significant spread in 238U/235U ratios, possibly related to redox conditions in the groundwater. This data is being evaluated in the context of other geochemical and isotopic data from this site. Leaching experiments are being carried out with undergraduates to determine the source and mobility of trace metals (including uranium) that are activated by high-CO2 groundwaters. This experiment will constitute a complete multi-isotope (U, Sr, and C) set of data on these natural analogue samples.

During July 2012, the Carbo QC method was tested for analysis of IC using additional buffer types, different buffer concentrations, and different IC concentrations. Preliminary tests were also conducted that use the addition of an organic modifier (methanol) in the analysis; the preliminary data was very encouraging and continued testing of that approach is planned.

A water-sampling trip to “Fe Falls” near Industry, Pennsylvania, occurred on July 24. Field analysis was completed for meter parameters, ferrous Fe, CO2, IC, and discharge. Samples were collected for cations and anions. The site has undergone some physical changes since the last visit. The mine water is now discharging from a hole adjacent to the “sealed portal.” The field data (alkalinity and ferrous Fe) were significantly higher than previously measured at the site.

The design stage of the experiments that will be used to predict the effect of increasing salt concentration on the partitioning of organic compounds from scCO2 began in June and finished in August. It was decided that the initial round of experiments would only involve the partitioning behavior of naphthalene. Naphthalene is a small PAH and is semi-volatile, which made it a good candidate for further evaluation. There is also previous experimental data that can be used to compare naphthalene partitioning data. The modeling of solid solubility in scCO2 using the Peng-Robinson equation of state was done in August. The model, done in Matlab, was finished. A new model, called an isothermal flash calculation, will be used to determine the K-factors of any solid between water and scCO2.

### Subtask 5.2 Optimization of Surface-Based Monitoring Networks

Near surface monitoring of potential leakage of CO2 is challenging because of the potentially large area of interest and the high potential for background.

Key Technology Accomplishments and Successes

The simulations were completed for near surface gas migration at the San Juan Basin site (South West Regional Partnership). Based on the results, with the topography considered and the different leaky parts for the wells, the conclusion is that CO2 seepage first appears at the banks of the hills, and is hardly detectable at the well head. For the total leakage rate considered (1.0 e-4 kg/s), the seepage fluxes at the surface are well below the values distinguishable from the background. However, they are not out of the detection capability range of the instruments. 
Based on this, and the overall seepage variation in the field, it may be better to perform monitoring not based only on point measurements but also on the overall pattern of seepage variation in the field.

Simulations for the pressure-history effect on the seepage anomalies at the Mammoth Mountain HSLTK were performed. The pressure-history (temporal variation) effect was found to be much more effective than the wind-topography interaction (spatial variation) effect in the current settings, but that both effects are detectable.

Laboratory experiments on retention of tracers under both moist and dry conditions were completed. The sample of coal which was chosen as a high-organic-material-containing geologic media displayed lower retention of the perfluorocarbon tracer (PFT) than the previously studied clay sample at a similar tracer/media ratio. No retention was observed under moist conditions. 
At a relative humidity of 30%, the lowest achievable reliably in the system, the PFT and helium co-eluted from the coal. The helium is co-introduced with the PFT as inert gas that measured the void volume of the column. The observed inhibition of adsorption on the coal by moisture is consistent with previous results obtained using inorganic media. In another study, the previously reported irreversible adsorption under dry conditions could not be reproduced during Q4 using the new quadrupole mass spectrometer (QMS). The better detection limit of the current QMS clearly shows that desorption of the PFT under dry conditions produces a long tail on the desorption curve lasting up to several days. The revised interpretation states that the introduction of moisture caused a sudden release of the reversibly adsorbed PFT which produced a peak in the elution curve. This peak was previously interpreted to be evidence for irreversible adsorption. It is now clear that the peak was an artifact of the continuing desorption of the reversibly retained compound that could not be seen because of the poorer sensitivity of the older QMS.

### Subtask 5.3 Development of Technology to Monitor CO2 and Pressure Plume

This task is aimed at using existing geophysical technology to develop a reliable technique to track CO2 and pressure plumes in the deep subsurface. Models and laboratory experiments are used to enhance interpretation of field data for passive and active seismic monitoring as well as surface deformation measurements.

Key Technology Accomplishments and Successes

An indentation test was performed on a sample of Marcellus Shale previously provided by NETL. Then instead of creating a wedge-shaped failure as the Berea sandstone samples had, the shale created a compact section of rock beneath the indenter that caused the surrounding rock to break off in layers around the indenter. These failures were in tension; however, the shale did not lose structural soundness all the way through the sample because of the failure. The previously tested Berea sandstone samples broke apart completely down to the base of the sample while the shale only broke halfway down the height of the sample. This result is believed to be because of the characteristics of the shale spreading out the force within the sample. The Berea sample failed at 227 kN (51,000 lbf) while the Marcellus sample was loaded to 103 kN (23,250 lbf).

Two laboratory tests were conducted to monitor induced seismicity at the laboratory scale, in addition to the six prior tests. The previous tests had monitored induced seismicity in a Berea sandstone sample that was loaded to failure using an indentation platen. During January, a different approach was used which simulated a strike-slip fault (mode I fracture). Unfortunately, this approach resulted in sample failure with very little plastic deformation, i.e., very few induced seismic events. For this reason, we returned to the initial approach of using the indentation platen. The final test was performed on a sample of Marcellus Shale previously provided by NETL. The test resulted in very good data quality which is currently being processed to determine seismic event locations. Following that step, double-difference tomography will be performed and results will be compared to PFC3D modeling.

Additional measurements of the carbonate EOR core were made using water and scCO2. In addition, one core has been saturated and then drained of heavy density (hydrocarbon approximate) fluids. The gain in weight suggests that irreducible fluids have remained in the core. This core is now being measured and the amount of fluid reflects approximately 5% of the pore space. This core will be flooded with scCO2, the rock properties measured, and then the fluid drained to determine the amount of fluid removed by CO2 saturation and draining. These results are being modeled with fluid replacement equations. In addition, the multiple azimuthal orientation measurements made earlier are being calculated, the Thomsen parameters for the two carbonate cores.

An extensive Geographic Information System (GIS) database for the Cranfield CO2 EOR site was constructed. This database is in ArcGIS 10 format and is presently 166 gigabytes in size. This size can be reduced by focusing only on the county within which the data was collected. Additional 3D reflection seismic data was also reviewed and detailed attribute interpretations were communicated to the Lawrence Berkeley National Laboratory (LBNL). The data and interpretations are being reviewed.

The variation of CO2 viscosity is being applied to the variation in ultrasonic S wave pressurization and depressurization differences. This variation is being thought about as a factor; previously the bulk modulus and density only were considered as relevant to the measured ultrasonic SH and SV velocities. The possibility that certain geometries of pores with high viscosity scCO2 could dominate a response over a relatively narrow range of effective pressures, leading to a measurable phenomenon are being considered.

Ant-tracking post-stack seismic processing to identify fault and fracture zones were completed using the In Salah dataset. These were estimated using two overall work flows which continued to be used for the seismic volume. In the first method, the 3D input seismic data was preconditioned using variance to determine a fault patches dataset. This data was then used as input to calculate an Ant-track volume. In the second method, the input seismic data was preconditioned by calculating the chaos attribute, this was used to determine a fault patches set and then an 
Ant-track volume. These methods are now being compared. Fault surfaces have been extracted from the Ant-track volume. These are being analyzed above, below, and within the damaged interval at In Salah.

### Subtask 5.4 Geophysical Methods to Detect Fracture Zones

The focus of this task is on developing and applying quantitative 3D seismic attribute technologies to storage formation property and fracture network modeling methods (such as FRACGEN). An integration of the new seismic attributes as the model drivers in FRACGEN fracture network modeling process will lead to the world-leading seismic attribute-assisted modeling methodologies, workflows, and practices that can be used in different CO2 storage projects.

Key Technology Accomplishments and Successes

A new seismic attribute denoted “Flexure” was developed that helps to better delineate both seismic scale and sub-seismic scale fractures, which is an improvement over the “Curvature” attribute that was widely used to define the faults and fractures.

There has been work on a geophysics conceptual model that can be used to detect sub-seismic scale fracture intensity and orientation using seismic waveform (amplitude) data. Based on this analysis, seismic AVO, AVAz, and Spectral Decomposition (AVF) attributes are suggested to be used to detect geologic texture that can be indicative of fracture intensity and orientation. This newly proposed concept could pose critical challenges to conventional practices and open a new avenue to detect fractures from seismic waveform attributes.

From a high-quality 3D seismic survey in a fractured reservoir in the Fort Worth basin onshore Texas (southern United States), limited angle stack amplitude in the faulted areas was found to be different from that in the unfaulted areas. Furthermore, using limited-azimuth data in the same fractured reservoir,  the amplitude at an azimuth (90°-120°) perpendicular to the fault was found to be stronger than that at an azimuth (0°-30°) parallel to the fault. These findings suggest that AVO, AVAz, and Spectral Decomposition data can find new applications to detect sub-seismic scale fractures. These results, which remain to be further tested and substantiated by more well bores and experiments, will have important implications for the evaluation of CO2 storage capacity and migration pathways and retention permanence in fractured and brine-saturated reservoirs.

A new C++ algorithm to computer 3D seismic curvature ad flexure attribute was developed using a new innovative complex trace method. This new method can save computational time by 400%, making the algorithm computationally more efficient and accurate compared with the conventional method. This algorithm can be used in combination with the waveform 
de-convolution and waveform model regression algorithm to better detect fractures.

## Task 6.0 Geospatial Data Resources

### Subtask 6.1 Atlas Development and NATCARB

Under this task NETL-ORD will maintain and grow NATCARB as a national geospatial data management and assessment portal. This work includes development of advanced tools and layers for geospatial display, interpretation, and analysis of storage datasets, including a new NATCARB viewer using a user-friendly MapViewer interface and development of Geodatabase templates to facilitate the collection and exchange of data for sources and storage formations. In addition, work under this task shall include supporting the development of the next edition of the Carbon Storage Atlas IV.

Key Technology Accomplishments and Successes

NETL-ORD is interested in maintaining the NATCARB database as a national geospatial data management and assessment portal for carbon sources, carbon sinks, and the transportation networks that connect them. The usefulness of NATCARB will grow by developing advanced tools and layers for geospatial display, interpretation, and analysis of storage datasets. Improved tools include a new NATCARB viewer with a user-friendly MapViewer interface, and development of Geodatabase Templates to facilitate the collection and exchange of data about sources and storage formations. The data gathering and display activities developed under this task are directly applicable to supporting the production of the next edition (Edition IV) of the Carbon Storage Atlas of the United States.

The NATCARB database was updated with data supplied by RCSPs to NATCARB on  March 30, 2012. Tim Carr and Maneesh Sharma at WVU were diligent about following up with the regional partnerships. Some of the data had boundary issues at RCSP borders, which required careful matching. State agency data was included that had not been in the previous edition of the Atlas or the database. Database query, display, and download capabilities were developed for NATCARB as a utility targeted at technical users.

The new NATCARB interface was rolled out at the SCC Carbon Storage Review meeting in Pittsburgh in August. A demonstration screen was set up in the exhibit/poster hall, and nearly everyone at the meeting came by to look at it. Reviews were generally favorable, especially for ease of use and ability to find things. NATCARB will be searchable and reachable through the EDX data warehouse being developed by NETL. These features will add to its usefulness and accessibility to the geologic, engineering, and environmental community.

The data for the new version of the North America Carbon Atlas (Atlas IV) is now complete and is being reviewed by NETL and contractors involved in the technical aspects of NATCARB. In general, RCSPs have provided data in agreed upon format. Atlas assembly has been delayed, but the paper and online versions are ready to be finalized. The prototype viewer is ready and has been tested. The viewer has new search and display functionality.

### Subtask 6.2 Geodatabase Development in Support of Onshore and Marine Storage Research, Risk, and Environmental Assessments

The goal of this task is to develop databases and a GIS project to evaluate risk, safety, and environmental issues associated with storage in geologic and geo-engineered systems. This task incorporates elements of Task 4.0 and Subtask 6.1; however, this task is expected to include base datasets from a breadth of resources, government, industry, commercial, as well as NETL-generated. Datasets will only be added under Carbon Storage’s funding that will be useful in some way to carbon storage research or to a carbon storage project. The base-datasets can be used for in-house analysis and interpretation, allowing the inclusion of proprietary datasets that can be used in support of SCC carbon storage research, risk and environmental analysis, and interpretation but which may not be viable for public release. Derivative products (maps, cross sections, etc.) from this geodatabase will be published to the NATCARB Atlas, external GIS portal, and/or a web-portal, as appropriate.

Key Technology Accomplishments and Successes

Work focused on coordinating with the regional partnerships, as well as working with WVU, on acquiring and gathering datasets needed for NATCARB as well as the NETL Energy Data Warehouse. Integration of the NATCARB database into the EDX was discussed and agreed upon. Quarterly updates to the NATCARB database will continue to be coordinated by WVU/Carr. However, these updates to the NATCARB database will also be incorporated into the EDX on a quarterly basis. The prior vintage of the NATCARB database was integrated into Version 1 of EDX because of the delays in the updated version of the NATCARB database. As soon as the updated version is released the offering in EDX will be updated.

- Work with the Information Technology and Engineering Support (ITES) subcontractor on the development of the NETL Energy Data eXchange, including the metadata protocols, governance, user interface, database architecture, and other organizational and technology development needs are progressing on schedule. The Use Cases and User Work Flow documents were finalized. Work with the ITES subcontractor on the development of the NETL EDX, culminated in the release of the full EDX Version 1 online in July 2012 at http://edx.netl.doe.gov. The appropriate security certificate for this website was updated by NETL’s cyber security group. A revised version of the factsheet was released. In response to guidance from SCC, and building on the ongoing collaboration with GWPC, GWPC and the EDX team have conferred on potential solutions for the states in relation to Class VI injection well data tracking and response. This effort has included coordination with EPA as well. Finally, advanced functionality, including the ability to define secure groups within EDX to allow for sharing of datasets with group members is underway. In FY13 these advanced functionalities will continue to evolve.

# TECHNOLOGY TRANSFER

Balashov, V.N., Guthrie, G.D., Hakala, A.J., Lopano, C.L., Rimstidt, D.J., and Brantley, S.L., “Predictive Modeling of CO2 Sequestration in Deep Saline Sandstone Reservoirs: Impacts of Geochemical Kinetics,” submitted to Applied Geochemistry.

Brennan, S.T., Burruss, R.C., Merrill, M.D., Freeman, P.A., and Ruppert, L.F., “A Probabilistic Assessment Methodology for the Evaluation of Geologic Carbon Dioxide Storage,” 2010.

Carbon Sequestration Leadership Forum (CSLF) Taskforce, “Estimation of CO2 Storage Capacity in Geological Media,” 2007.

Crandall, D., “Determining Interactions between CO2 and Basalt with Three-dimensional Imaging,” Presentation at United States Geological Survey Workshop on CO2 Sequestration in Unconventional Reservoirs, Shepherdstown, WV, March 28–29, 2012.

Dilmore, R., “Experimental Characterization and Reservoir Simulation of CO2 Storage in and Enhanced Gas Recovery from Organic-Rich Shale Formations,” Presentation at the USGS Geologic CO2 Sequestration in Unconventional Reservoirs Workshop, Shepherdstown, WV, March 28–29, 2012.

Gao, D., Wilson, T., Zhu, L., and Marfurt, K.J., “3D Seismic Curvature and Flexure for Fractured Reservoir Characterization at Teapot Dome (Wyoming)” prepared for presentation at the 31st GCSSEPM foundation Bob F. Perkin research conference.

Goodman, A., “A Review of National CO2 Sequestration Assessments,” Presentation at the USGS Geologic CO2 Sequestration in Unconventional Reservoirs Workshop,  Shepherdstown, WV, March 28–29, 2012.

Karamalidis, A., Griffith, C., Dilmore, R., Hedges, S., Ostertag-Henning, C., and Goodman, A., “Geochemical Reactions of Illite, Labradorite, and Siderite with CO2-Saturated Salt Solution under Geologic CO2 Storage Conditions,” 243rd ACS National Meeting, San Diego, CA.

Popova, O.H., Karimi, B., Small, M., McCoy, S., Thomas, A., and Morgan, G., “The Size of Available CCS Reservoirs: Uncertainty Analysis of Geologic Storage Resource Estimates for CO2 Sequestration in Saline Formations,” STG conference ‘Science and Technology in Society,’ STGlobal Consortium, American Association for the Advancement of Science (AAAS), and The National Academies, Washington, DC, March 30-31, 2012.

Popova, O.H., Small, M., McCoy, S., Thomas, A.C., and Morgan, G., “CO2 Sequestration in Deep Saline Formations. How Much Storage Resource Do We Have?” presented at Advisory Committee meeting, Carnegie Mellon Electricity Industry Center, Pittsburgh, PA, October 20, 2011.

Popova, O.H., Small, M., McCoy, S., Thomas, A.C., Karimi, B., Carter, K., and Goodman, A., “Comparative Analysis of Carbon Dioxide Storage Resource Assessment Methodologies,” published in Environmental Geosciences Journal, Vol 19, No. 3, September 2012, p105-124.

Spencer, L.K., Bradshaw, J., Bradshaw, B.E., Lahtinen, A., and Chirinos, A., “Regional Storage Capacity Estimates: Prospectivity Not Statistics,” 2010.

Viswanathan, H.S., Dai, Z., Keating, E.H., Hakala, J.A., Lopano, C.L., Carey, J.W., Zheng, L., and Pawar, R., “Mechanisms for Arsenic Release at the CO2 Sequestration Natural Analog Site in Chimayo, New Mexico,” presented at the American Geophysical Union Conference in San Francisco, CA, December 2011.

Viswanathan, H.S., Dai, Z., Lopano, C.L., Keating, E.H., Hakala, J.A., Scheckel, K.G., Zheng, L., Guthrie, G.D., and Pawar, R., “Developing a Robust Geochemical and Reactive Transport Model to Evaluate Possible Sources of Arsenic at the CO2 Sequestration Natural Analog Site in Chimayo, New Mexico,” revisions submitted to International Journal of Greenhouse Gas Control, March 29, 2012.

The National Energy Technology Laboratory (NETL) conducts cutting-edge energy research and technology development and analyzes energy systems and international energy issues for the U.S. Department of Energy. The NETL Regional University Alliance (NETL-RUA) is an applied research collaboration that combines NETL’s energy research expertise with the broad capabilities of five nationally recognized, regional universities: Carnegie Mellon University (CMU), The Pennsylvania State University (Penn State), University of Pittsburgh (Pitt), Virginia Tech, and West Virginia University (WVU), and the engineering and construction expertise of an industry partner (URS). NETL-RUA leverages its expertise with current fossil energy sources to discover and develop sustainable energy systems of the future, introduce new technology, and boost economic development and national security.

Jared Ciferno

Director

SCC, Office of Coal and Power R&D

DOE-NETL

Robert Romanosky

Deputy Director

SCC, Office of Coal and Power R&D

DOE-NETL

Traci Rodosta

Technology Manager, Carbon Storage

SCC, Office of Coal and Power R&D

DOE-NETL

Cynthia Powell

Director, ORD

DOE-NETL

Timothy McNulty

Associate Vice-President for Government Relations, CMU

Henry Foley

Vice President for Research, Penn State

George Klinzing

Vice Provost for Research, Pitt

Robert Walters

Vice President for Research, Virginia Tech

Fred King

Vice President for Research and Economic Development, WVU

Kevin Donovan

RES Program Manager, URS Corporation